MxSense IoT Monitoring System

Cloud Architecture using AWS IoT Core

1.Introduction
The MxSense system is a multi-sensor electronic sensing platform designed to collect environmental, chemical, and visual data from samples. The device integrates several sensing channels including environmental sensors, VOC sensors, gas sensors, vision modules, and particulate sensors.
The sensor data collected from the device is transmitted to the cloud using AWS IoT services. AWS IoT Core receives the data and enables secure communication between the device and cloud services.The cloud platform processes incoming data, stores it for analysis, detects abnormal conditions, and generates alerts for monitoring applications. This architecture enables real-time monitoring, data logging, and intelligent analysis of sensor data.

2.Device layers
In the sensing system, every device is uniquely identified using a Device ID. This Device ID can either be derived directly from the MCU chip’s hardware ID or generated by the enterprise during device registration. Typically, one physical Esense device corresponds to one MCU, and the chip ID of the MCU acts as the base identifier for that device. This ensures that each device deployed in the field can be uniquely recognized, authenticated, and tracked within the platform or cloud system.

Each device measures multiple parameters using different sensors or cameras integrated into the system. A parameter refers to any measurable quantity such as temperature, humidity, gas concentration, particulate matter, image-based features, or other physical properties captured by the device. To systematically manage and process these measurements, every parameter is assigned a unique Parameter ID (Param ID). The Param ID acts as a reference for identifying which specific sensor or camera generated the data.

This structured mapping allows the system to clearly associate every measurement with the correct sensing module, making it easier to store, analyze, and interpret the data across different channels of the device architecture.

Such an identification scheme is important because the device collects data from multiple sensing channels such as environmental sensors, gas sensors, vision systems, or particulate monitors and each of these must be individually traceable within the platform for accurate data processing, logging, and analysis.

The sensors used in the first prototype include:

- Environmental Sensors

- SGP41 – VOC / air quality sensor

- BME688 – Gas, temperature, humidity, and pressure sensor

- SHT45 – High precision temperature and humidity sensor

- BMP280 – Barometric pressure sensor

- Gas Detection Sensors

- Electrochemical gas modules for CO, NO₂, SO₂, H₂S, NH₃, Cl₂, O₃ and other gases

- Particulate Monitoring

- PMS7003 – PM1.0, PM2.5 and PM10 particulate matter sensor

- Flow and Sampling Control

- MPXV7002DP – Differential pressure sensor for airflow monitoring

- 12V micro air pump for sample airflow

- Solenoid valve for air path control

- PTFE tube for controlled sampling

- Optical and Imaging Sensors

- ESP32-CAM – basic imaging module

- Pi Camera V2 – high resolution imaging

- Arducam B0462 – camera module for Arduino Giga

- MLX90640 – thermal imaging sensor

- Spectral Sensor

- AS7341 – spectral color sensor for optical analysis

- Signal Conditioning and Monitoring

- INA219 – current and power monitoring sensor

- TCA9548A – I²C multiplexer for connecting multiple sensors

parameter_id = sensor name + measured parameter

| Category | Sensor / Source | Parameter | Parameter ID |
| --- | --- | --- | --- |
| Environmental | SGP41 | VOC Index | sgp41_voc_index |
| Environmental | SGP41 | Raw VOC Signal | sgp41_voc_raw |
| Environmental | SGP41 | NOx Index | sgp41_nox_index |
| Environmental | BME688 | Temperature | bme688_temp |
| Environmental | BME688 | Humidity | bme688_humidity |
| Environmental | BME688 | Pressure | bme688_pressure |
| Environmental | BME688 | Gas Resistance | bme688_gas_resistance |
| Environmental | BME688 | IAQ Index | bme688_iaq |
| Environmental | SHT45 | Temperature | sht45_temp |
| Environmental | SHT45 | Humidity | sht45_humidity |
| Environmental | BMP280 | Pressure | bmp280_pressure |
| Environmental | BMP280 | Temperature | bmp280_temp |
| Gas Detection | Electrochemical | Carbon Monoxide | gas_co_ppm |
| Gas Detection | Electrochemical | Nitrogen Dioxide | gas_no2_ppm |
| Gas Detection | Electrochemical | Sulfur Dioxide | gas_so2_ppm |
| Gas Detection | Electrochemical | Hydrogen Sulfide | gas_h2s_ppm |
| Gas Detection | Electrochemical | Ammonia | gas_nh3_ppm |
| Gas Detection | Electrochemical | Chlorine | gas_cl2_ppm |
| Gas Detection | Electrochemical | Ozone | gas_o3_ppm |
| Gas Detection | Electrochemical | Sensor Temperature | gas_sensor_temp |
| Gas Detection | Electrochemical | Sensor Status | gas_sensor_status |
| Particulate | PMS7003 | PM1.0 | pm1_0 |
| Particulate | PMS7003 | PM2.5 | pm2_5 |
| Particulate | PMS7003 | PM10 | pm10 |
| Particulate | PMS7003 | Particle Count >0.3µm | particle_0_3um |
| Particulate | PMS7003 | Particle Count >0.5µm | particle_0_5um |
| Particulate | PMS7003 | Particle Count >1µm | particle_1um |
| Flow Control | MPXV7002DP | Differential Pressure | airflow_dp |
| Flow Control | Derived | Airflow Rate | airflow_rate |
| Flow Control | Pump | Pump Status | pump_status |
| Flow Control | Pump | Pump Speed | pump_pwm |
| Flow Control | Valve | Valve State | valve_state |
| Flow Control | Sampling | Sampling Mode | sampling_mode |
| Optical | Camera | Image Frame ID | image_frame_id |
| Optical | Camera | Image Timestamp | image_timestamp |
| Optical | Camera | Image Brightness | image_brightness |
| Optical | Vision Model | Defect Score | vision_defect_score |
| Optical | Vision Model | Visual Grade | vision_grade_score |
| Thermal Imaging | MLX90640 | Max Temperature | thermal_temp_max |
| Thermal Imaging | MLX90640 | Min Temperature | thermal_temp_min |
| Thermal Imaging | MLX90640 | Avg Temperature | thermal_temp_avg |
| Spectral | AS7341 | Channel 1 Intensity | spectral_ch1 |
| Spectral | AS7341 | Channel 2 Intensity | spectral_ch2 |
| Spectral | AS7341 | Channel 3 Intensity | spectral_ch3 |
| Spectral | AS7341 | Channel 4 Intensity | spectral_ch4 |
| Spectral | AS7341 | Channel 5 Intensity | spectral_ch5 |
| Spectral | AS7341 | Channel 6 Intensity | spectral_ch6 |
| Spectral | AS7341 | Channel 7 Intensity | spectral_ch7 |
| Spectral | AS7341 | Channel 8 Intensity | spectral_ch8 |
| Spectral | Model Output | Spectral Fingerprint | spectral_fingerprint |
| Spectral | Model Output | Adulteration Score | spectral_adulteration_score |
| Power Monitoring | INA219 | Bus Voltage | power_voltage |
| Power Monitoring | INA219 | Current | power_current |
| Power Monitoring | INA219 | Power Consumption | power_consumption |
| Device Health | System | Device Temperature | device_temp |
| Device Health | System | CPU Usage | cpu_usage |
| Device Health | System | Memory Usage | memory_usage |
| Device Health | System | Storage Remaining | storage_remaining |
| Device Health | System | Device Health Status | device_health |
| Device Health | System | Firmware Version | firmware_version |

Data : Collection - > Acquiring - > Managing - > Training - > Inferencing

3.Data Management

i. Data Collection- Data collection refers to the process in which sensors measure physical or environmental parameters from the sampling environment. The MxSense sensing platform integrates multiple sensor modules such as environmental sensors, gas sensors, particulate sensors, flow monitoring sensors, spectral sensors, and imaging modules. These sensors continuously observe the conditions inside the sampling chamber and generate raw measurements corresponding to physical quantities such as temperature, humidity, gas concentration, particulate matter levels, airflow, and spectral characteristics. These measurements represent the primary data produced by the sensing hardware before any digital processing occurs.

ii. Data Acquisition
Data acquisition is the process by which the device controller collects sensor measurements and converts them into structured digital data. The microcontroller unit (MCU) reads sensor outputs using communication interfaces such as I²C, UART, SPI, or analog-to-digital converters (ADC). The controller gathers readings from all connected sensors and synchronizes them into a unified dataset.

Before transmission, the device performs basic preprocessing operations such as:

- Timestamping each measurement

- Filtering noise from sensor readings

- Converting raw values into standard units

- Combining measurements from multiple sensors into a structured telemetry dataset

Once preprocessing is completed, the controller packages the processed sensor readings into a telemetry message (typically JSON format). This message represents the digital representation of the collected sensor data that can be transmitted to the edge layer for further processing.

iii. Data Sampling 
Data sampling defines the timing and frequency at which sensor measurements are captured and transmitted by the device. The controller schedules sensor readings at predefined intervals or during specific operational stages of the sensing process. For example, sensors may be sampled periodically during continuous monitoring or triggered during specific phases of a sampling workflow.

In addition to numerical sensors, sampling also applies to vision sensors where the controller triggers image capture at defined intervals or during specific sampling events. This ensures that data is collected consistently across different sensing channels. The sampled measurements are then prepared for transmission to the edge server, which acts as a gateway between the sensing device and the cloud infrastructure. 

Timestamping of Numerical Data

Each numerical sensor measurement generated by the device is associated with a timestamp to indicate the exact time at which the data was recorded. The device clock is synchronized using Network Time Protocol (NTP) or wireless time synchronization mechanisms to maintain accurate system time across all devices in the network. When numerical sensor data is collected from the sensors, the controller attaches the current synchronized timestamp to each measurement record. This ensures that all telemetry data transmitted to the edge server and cloud infrastructure can be correctly ordered, analyzed over time, and correlated with other sensor readings or system events. Timestamping is essential for time-series storage, event detection, and historical analysis of sensor data.

Signature of Data

Each set of collected sensor readings is represented as a structured data record containing a timestamp and the corresponding values of all measured parameters. The data signature can be represented as either an array structure or a JSON object, depending on the communication protocol and system implementation. In most cases, JSON format is used because it is lightweight, human-readable, and easily supported by IoT messaging systems such as MQTT and cloud APIs.

- Numerical sensor data

Step 1 – Sensor Measurement-Environmental, gas, particulate, flow, spectral, and electrical sensors measure physical parameters from the sampling environment.

Step 2 – Data Acquisition by Controller-MCU reads sensor values using communication interfaces such as I²C, UART, or analog-to-digital converters. The controller gathers readings from all connected sensors.

Step 3 – Data Pre-Processing-Before transmission, the device performs basic preprocessing operations. These may include timestamping measurements, filtering noise, converting units, and combining readings from multiple sensors into a unified dataset.

Step 4 – Telemetry Packet Creation-The processed sensor readings are packaged into a telemetry message(JSON). 

Step 5 – MQTT Client Transmission to Edge Server-An MQTT client running on the device publishes the telemetry packet to the edge server. The MQTT client establishes a secure connection and publishes the data to a predefined MQTT topic such as:

Step 6 – Network Communication to Edge Layer-The telemetry message is transmitted over the local network (Wi-Fi or Ethernet) from the device to the edge server, which acts as a gateway between the sensing devices and the cloud infrastructure.

Step 7 – Edge Server Processing and Forwarding-The edge server receives the telemetry message and may perform intermediate tasks such as buffering, filtering, aggregation, or validation. After processing, the edge server forwards the telemetry data to the AWS cloud using MQTT or HTTPS.

Step 8 – Message Reception by AWS IoT Core-AWS IoT Core receives the forwarded telemetry messages from the edge server and acts as the MQTT broker for the cloud infrastructure. It manages device communication and securely routes the incoming data to downstream AWS services for logging and processing.

B. Vision data 

Step 1 – Image Capture-The vision sensor captures an image of the sample placed inside the sensing chamber.

Step 2 – Image Acquisition by Controller-The MCU triggers the camera to capture an image at defined intervals or during specific sampling stages. The captured image is transferred from the camera module to the controller.

Step 3 – Temporary Image Storage-The captured image is temporarily stored in the device memory or local storage of the controller. This allows the device to prepare the image for further transmission to the cloud.

Step 4 – Image Pre-Processing (Optional)-The device may perform basic preprocessing operations such as resizing, compression, or format conversion to reduce the file size and optimize transmission efficiency.

Step 5 – Image Transmission to Edge Server-The processed image is transmitted from the device to the edge server using the local network (Wi-Fi or Ethernet). The edge server acts as an intermediate gateway between the sensing device and the cloud infrastructure.

Step 6 – Image Upload to Cloud Storage-The edge server uploads the received image to cloud storage using secure HTTP or HTTPS communication. In the AWS architecture, the image is stored in Amazon S3 object storage.

Step 7 – Metadata Creation and Transmission-After the image is uploaded, the system generates metadata describing the captured image. This metadata may include the device ID, camera type, timestamp, image identifier, and the storage location of the image in Amazon S3. The metadata is transmitted as a small message to notify the cloud system that a new image has been collected and stored.

Step 8 – Completion of Data Collection-Once the image is stored in Amazon S3 and the corresponding metadata is transmitted, the vision data collection process is complete. The stored image can then be used for further processing, analysis, or visualization by downstream systems.

4.Data Retention 

1. Numerical Sensor Data

Numerical telemetry data generated by environmental sensors, gas sensors, particulate sensors, and airflow sensors is stored in the cloud database. These measurements are generated at high frequency and are primarily used for real-time monitoring and historical analysis.

| Data Type | Retention Period |
| --- | --- |
| Raw sensor telemetry | 90 days |
| Aggregated sensor summaries | 1–2 years |

After the initial retention period, raw data may be aggregated or archived to reduce storage requirements while preserving long-term trends.

2. Vision Data
Images captured by the camera modules are large files and therefore require controlled retention.

| Data Type | Retention |
| --- | --- |
| Raw images | 60 days |
| Important or flagged images | 1 year |

Stored in: Amazon S3-Using S3 lifecycle policies, older images can automatically be deleted or moved to archival storage.

3. Image Meta Data
Metadata describing captured images includes:

- device ID

- camera type

- timestamp

- image location in S3

Stored in: Amazon DynamoDB-Metadata storage allows efficient querying of images based on device, timestamp, or camera source.

| Data Type | Retention |
| --- | --- |
| Image metadata | 3 years |

4. Event and Alert Logs

Event logs record abnormal conditions such as threshold violations or system alerts.Stored in: CloudWatch logs

| Data Type | Retention |
| --- | --- |
| Event logs | 1 year |
| Critical system alerts | 3 years |

# 5.System Monitoring Logs

System-level logs record information such as device connectivity, data transmission status, and system health.Stored in: AWS CloudWatch

| Data Type | Retention |
| --- | --- |
| System monitoring logs | 6 months |

Data Refresh Policy

The data refresh policy defines the frequency at which sensor measurements, images, and system status information are updated and transmitted to the cloud infrastructure. The refresh intervals are selected to ensure timely monitoring while minimizing unnecessary network traffic and processing load.

| Data Type | Refresh Frequency |
| --- | --- |
| Environmental sensor data | 10 seconds |
| Gas detection data | 10 seconds |
| Particulate sensor data | 15 seconds |
| Airflow and sampling control data | 5 seconds |
| Spectral sensor data | 30 seconds |
| Vision image capture | 60 seconds or event-based |
| Edge aggregation update | 1 minute |
| Dashboard refresh | 10–15 seconds |
| Event detection evaluation | real-time |

5.Data Logging
   A. Numerical sensor data 

Step 1 – Message Reception by AWS IoT Core-AWS IoT Core receives telemetry messages forwarded from the edge layer. It acts as the cloud MQTT broker and securely manages incoming device communication.

Step 2 – Message Processing by IoT Rules Engine-The AWS IoT Rules Engine evaluates incoming MQTT messages and determines how the data should be routed within the cloud infrastructure.

Step 3 – Data Formatting and Validation-Before storage, the incoming data may be validated and formatted to ensure consistency, including verifying timestamps and organizing records.

Step 4 – Storage in Time-Series Database-The validated telemetry data is stored in a time-series database such as Amazon Timestream.

Step 5 – Data Indexing and Organization-The stored data is indexed based on attributes such as device identifier and timestamp.

Step 6 – Persistent Data Storage-Once stored, the telemetry data becomes available for dashboards, analytics, event detection, and monitoring systems.

### B.Vision Data Logging

Step 1 – Image Storage in Amazon S3-Captured images are stored as objects in an Amazon S3 bucket. Each image is assigned a unique object key to ensure that new uploads do not overwrite existing files. Amazon S3 provides scalable and durable storage for large files such as images.

Step 2 – Reception of Metadata Message-The metadata message describing the captured image is received by AWS IoT Core through the configured MQTT topic. This message contains information such as device ID, camera type, timestamp, and the S3 storage location of the image.

Step 3 – Metadata Routing via IoT Rules Engine-The AWS IoT Rules Engine processes the incoming metadata message and routes it to downstream storage services according to predefined rules.

Step 4 – Metadata Storage in Database-The metadata associated with each image is stored in a cloud database such as Amazon DynamoDB. Storing metadata separately allows efficient indexing and querying of images based on attributes such as device ID, timestamp, or camera type.

Step 5 – Persistent Metadata Storage-Once stored in the database, the image metadata becomes persistently available for downstream applications such as monitoring dashboards, analytics services, and event detection systems.

       6.Data Processing
	A.Common Edge processing
Step 1 – Reception of Data from Devices-The edge server receives telemetry data and image data transmitted from sensing devices through the local network using communication protocols such as MQTT or HTTP.

Step 2 – Data Validation-The edge server verifies the integrity of incoming data messages by checking parameters such as message structure, device identifier, and timestamps to ensure data consistency.

Step 3 – Data Buffering and Temporary Storage-Incoming data may be temporarily stored on the edge server to prevent data loss in case of temporary network interruptions between the edge layer and the cloud infrastructure.

Step 4 – Forwarding Data to Cloud Services-After validation and buffering, the edge server forwards the data to the appropriate AWS services. Numerical telemetry data is typically forwarded to AWS IoT Core, while image data is forwarded to Amazon S3.

B.Numerical Data processing at Edge
Step 1 – Sensor Data Reception-The edge server receives numerical telemetry data from devices in the form of structured messages.

Step 2 – Data Filtering and Noise Reduction-The edge server may apply basic filtering techniques to remove noise or irregular values from the sensor readings.

Step 3 – Data Aggregation-Multiple sensor readings may be aggregated over a short time interval to reduce data volume and improve stability of measurements.

Step 4 – Data Forwarding to Cloud-The processed numerical data is forwarded to AWS IoT Core, where it is further routed to cloud storage and analytics services

C.Vision Data processing at Edge

Step 1 – Image Reception-The edge server receives captured images transmitted from the sensing device.

Step 2 – Image Optimization-Basic preprocessing operations may be performed on the image data such as resizing, compression, or format conversion to reduce file size and improve transmission efficiency.

Step 3 – Image Upload to Cloud Storage-After optimization, the edge server uploads the image to Amazon S3 object storage using secure communication protocols.

Step 4 – Metadata Forwarding-Metadata describing the captured image, such as device ID, timestamp, and storage location, is forwarded to AWS IoT Core for logging and indexing.

D.Cloud Data processing
Step 1 – Cloud Data Reception-AWS IoT Core receives messages forwarded from the edge server. It acts as a managed MQTT broker and securely manages communication between the edge layer and AWS cloud services.

Step 2 – Rule Evaluation-The AWS IoT Rules Engine evaluates incoming MQTT messages and determines how the data should be handled within the cloud infrastructure.

Step 3 – Data Routing Based on Data Type-Depending on the message topic and data type, the rules engine routes the data to appropriate AWS services:

- Numerical telemetry data is routed to processing services and then stored in Amazon Timestream.

- Vision image files are stored in Amazon S3.

- Image metadata messages are routed to Amazon DynamoDB for indexing and retrieval.

Step 4 – Processing using AWS Lambda-AWS Lambda functions are triggered by incoming data messages or storage events. These serverless functions perform additional processing tasks such as validating incoming data, transforming message formats, computing derived values, or analyzing metadata associated with captured images.

Step 5 – Numerical Data Storage-Numerical sensor data is stored in Amazon Timestream, a time-series database optimized for storing timestamped sensor measurements and supporting efficient analytical queries.

Step 6 – Vision Data Storage-Captured images are stored in Amazon S3 object storage. Amazon S3 provides scalable and durable storage for large files such as images and other media.

Step 7 – Metadata Storage-Metadata describing captured images is stored in Amazon DynamoDB. This enables efficient indexing and querying of images based on parameters such as device identifier, timestamp, or camera type.

Step 8 – Cloud Monitoring-AWS CloudWatch monitors the performance and operational health of the cloud infrastructure. It records logs, tracks system metrics, and provides alerts for abnormal system behavior or processing failures.

	7.Policy Management

i.Data Retention and Data Refresh Policy

The Data Retention Policy defines the duration for which collected data is stored in the system before it is archived or removed. This policy ensures efficient storage utilization while maintaining sufficient historical data for analysis and monitoring.

The Data Refresh Policy defines the frequency at which sensor measurements, system status information, and dashboard data are updated. These refresh intervals ensure that the monitoring system receives timely updates without generating unnecessary network traffic.

Both retention and refresh policies are maintained as configuration parameters in the cloud system. The policy parameters are stored in a configuration database such as Amazon DynamoDB, allowing backend services and edge components to retrieve the policy settings dynamically. This centralized configuration approach enables consistent policy enforcement across all devices and services.

ii.AI Model Management Policy

The AI Model Management Policy defines operational rules related to the lifecycle of machine learning models used in the system. This includes policies for model training frequency, model update intervals, and inference execution.

The policy specifies how often models should be retrained using new datasets, how frequently updated models should be deployed to the system, and whether inference is performed at the edge layer or within the cloud environment. These policy parameters allow the system to maintain accurate and up-to-date models while ensuring efficient computational resource usage.

The configuration parameters related to AI model management are stored in the cloud configuration database such as Amazon DynamoDB. This enables system components to access the current model configuration, active model version, and update schedules.

iii.Model Storage
The trained AI models and related artifacts are stored separately from configuration policies. Model files, checkpoints, and training artifacts are stored in Amazon S3 object storage.

Using Amazon S3 provides scalable and durable storage for model assets and allows different components of the system to access the required model versions during training, deployment, or inference operations. Model versioning can also be maintained within the storage structure to support model updates and rollback if required.

	8.System Events 
The platform generates system events to track device operation, sensor measurements, data transmission, and system health. Events represent significant occurrences within the system and are used for monitoring, alert generation, and system diagnostics. These events are detected based on device activity, sensor readings, data processing stages, and operational conditions. The following categories summarize the key events considered in the system.

1. Device and Connectivity Events

- Device Startup-This event is generated when the sensing device powers on and begins its initialization process. It indicates that the system has started and the device firmware is loading operational components.

- Device Shutdown-This event occurs when the device stops operation due to power-off, system command, or system failure.

- Device Restart-This event is triggered when the device reboots, either intentionally through a system update or due to an unexpected system reset.

- Device Connected to Network-This event indicates that the sensing device has successfully established network connectivity with the local network or gateway.

- Device Disconnected from Network-This event occurs when the device loses its network connection due to connectivity failure or network interruption.

- Device Reconnected After Disconnection-This event indicates that the device has restored its connection to the network after a temporary communication failure.

- Device Registration to Cloud-This event is generated when a device successfully authenticates and registers with the cloud infrastructure for communication.

- Device Heartbeat Timeout-This event occurs when the system fails to receive periodic heartbeat messages from the device within the expected time interval, indicating possible device failure or connectivity issues.

2. Sensor Data Events

- Environmental Sensor Data Received-This event is generated when environmental sensors such as temperature, humidity, or pressure sensors produce new measurement data.

- Gas Sensor Data Received-This event indicates that gas detection sensors have produced new measurements related to gas concentrations.

- Particulate Sensor Data Received-This event occurs when the particulate matter sensor generates measurements for particulate concentrations such as PM1.0, PM2.5, or PM10.

- Spectral Sensor Data Received-This event indicates that the spectral sensor has captured spectral measurements used for optical or chemical analysis.

- Airflow Sensor Data Received-This event occurs when airflow sensors measure differential pressure or airflow conditions within the sampling chamber.

- Sensor Threshold Exceeded-This event is generated when a sensor reading exceeds a predefined upper threshold indicating abnormal or unsafe conditions.

- Sensor Reading Below Minimum Threshold-This event occurs when a measured parameter falls below the expected operating range.

- Sensor Data Anomaly Detected-This event indicates that abnormal or inconsistent sensor readings have been detected by anomaly detection algorithms.

3. Sampling and Flow Control Events

- Sampling Process Started-This event indicates that the system has initiated the sampling procedure for collecting sensor measurements.

- Sampling Process Completed-This event is generated when the sampling cycle finishes and all measurements for the sampling session are collected.

- Air Pump Started-This event occurs when the sampling air pump is activated to draw air or gas samples into the sensing chamber.

- Air Pump Stopped-This event indicates that the sampling pump has been deactivated after completing the sampling process.

- Airflow Below Expected Level-This event occurs when the measured airflow rate drops below the acceptable operating range.

- Airflow Above Expected Level-This event indicates that the airflow rate has exceeded the expected operating limits.

- Valve Opened-This event occurs when the sampling valve opens to allow air or gas to enter the sensing chamber.

- Valve Closed-This event indicates that the sampling valve has been closed to stop airflow into the system.

4. Vision System Events

- Image Capture Initiated-This event indicates that the camera system has been triggered to capture an image of the sample.

- Image Capture Completed-This event occurs when the camera successfully captures an image.

- Image Upload Started-This event is generated when the captured image begins transmission to the edge server or cloud storage.

- Image Upload Completed-This event indicates that the image has been successfully transferred and stored.

- Image Upload Failed-This event occurs when the image transmission process fails due to network or storage errors.

- Image Processing Completed-This event indicates that image preprocessing or analysis operations have been completed.

5. Data Transmission Events

- Telemetry Packet Created-This event occurs when sensor readings are packaged into a structured telemetry message for transmission.

- Telemetry Message Published-This event indicates that the device has published the telemetry message to the communication protocol (such as MQTT).

- Telemetry Message Received by Edge Server-This event occurs when the edge server successfully receives telemetry data from the sensing device.

- Telemetry Message Forwarded to Cloud-This event indicates that the edge server has forwarded telemetry data to cloud services.

- Telemetry Message Received by Cloud Services-This event occurs when the cloud platform receives incoming telemetry data.

- Data Transmission Failure-This event is generated when telemetry data cannot be successfully transmitted due to communication errors.

6. Data Processing Events

- Data Validation Completed-This event occurs after incoming sensor data has been validated to ensure correct structure and format.

- Data Aggregation Completed-This event indicates that multiple sensor readings have been combined into aggregated data values.

- Sensor Data Stored in Database-This event occurs when validated numerical telemetry data is stored in the cloud database.

- Image Metadata Stored in Database-This event indicates that metadata related to captured images has been stored for indexing and retrieval.

- Image Stored in Cloud Storage-This event occurs when captured images are successfully stored in cloud object storage.

7. Event Detection and Alert Events

- Threshold Violation Detected-This event occurs when sensor measurements exceed predefined thresholds.

- Sensor Anomaly Detected-This event indicates that abnormal patterns in sensor data have been detected through analysis.

- Abnormal Gas Concentration Detected-This event is triggered when gas sensor readings indicate unsafe gas concentration levels.

- High Particulate Level Detected-This event occurs when particulate matter levels exceed acceptable environmental limits.

- Environmental Condition Outside Acceptable Range-This event indicates that environmental parameters such as temperature or humidity fall outside the allowed range.

- Alert Generated-This event occurs when the system generates an alert in response to an abnormal condition.

- Alert Notification Sent-This event indicates that the alert message has been sent to users or monitoring systems.

- Alert Acknowledged by Operator-This event occurs when a system operator acknowledges receipt of the alert.

8. System Health and Maintenance Events

- Sensor Malfunction Detected-This event indicates that a sensor is not functioning correctly or has stopped responding.

- Sensor Calibration Required-This event occurs when a sensor requires calibration to maintain measurement accuracy.

- Storage Capacity Threshold Reached-This event indicates that available storage capacity has reached a predefined limit.

- Edge Server Unavailable-This event occurs when the edge server becomes unreachable or stops responding.

- Cloud Service Error Detected-This event indicates that a cloud service component has encountered an operational error.

9.Event detection

Event detection identifies significant conditions or anomalies in the collected data. The system continuously monitors incoming sensor data and metadata to determine whether predefined conditions or thresholds have been exceeded.

Step 1 – Data Monitoring-Processed sensor data stored in the cloud is continuously monitored for abnormal patterns, threshold violations, or predefined trigger conditions.

Step 2 – Event Evaluation-AWS IoT Events or AWS Lambda evaluates incoming data against predefined rules or thresholds. These rules determine whether a specific condition qualifies as an event.

Step 3 – Event Generation-When a predefined condition is satisfied, the system generates an event that represents the detected condition.

Step 4 – Event Forwarding-The generated event is forwarded to downstream services responsible for alert generation, notification, or automated system responses.


       10.Alerts and notifications

Step 1 – Event Trigger Reception-The event generated by the event detection system is received by an alert management component in the cloud infrastructure.

Step 2 – Alert Generation-Based on the type and severity of the event, the system generates an alert message describing the detected condition. This alert message may include information such as the device identifier, event type, timestamp, and relevant sensor values.

Step 3 – Notification Service Activation-The alert message is forwarded to a notification service such as Amazon Simple Notification Service (Amazon SNS). SNS enables the system to distribute alerts to multiple subscribers.

Step 4 – Alert Delivery-The notification service delivers alerts through various channels depending on system configuration. These channels may include:

- Email notifications

- SMS alerts

- Application notifications

- Web dashboard alerts

Step 5 – Alert Logging and Monitoring-All generated alerts are logged and monitored through services such as AWS CloudWatch to track system behavior and maintain operational visibility.

      11.Monitoring

Step 1 – Data Retrieval from Cloud Storage-The dashboard application retrieves stored data from AWS services. Numerical sensor data is queried from Amazon Timestream, while image files are retrieved from Amazon S3. Metadata associated with images is obtained from Amazon DynamoDB.

Step 2 – Backend API Integration-A backend service or API layer communicates with AWS services to fetch the required data and prepare it for display in the dashboard. This API processes queries and returns formatted data to the frontend application.

Step 3 – Data Visualization-The dashboard presents numerical sensor readings using visual elements such as charts, graphs, and tables. Time-series plots allow users to monitor changes in environmental and gas sensor values over time.

Step 4 – Image Display-Captured images stored in Amazon S3 are displayed on the dashboard to provide visual inspection of the sampled environment. The dashboard uses the stored image URLs and metadata to retrieve and present the latest images.

Step 5 – Alert Display and Notifications-Alerts generated by the event detection system are displayed on the dashboard interface. Users can view the alert details, including the event type, affected device, and timestamp.

Step 6 – System Monitoring Interface-The dashboard also provides system-level information such as device status, data collection activity, and recent events. This enables operators to monitor the health and performance of the sensing platform in real time.

12.AWS services

- Message routing → AWS IoT Rules Engine

- Numerical sensor data storage → Amazon Timestream

- Image storage → Amazon S3

- Image metadata storage → Amazon DynamoDB

- Data processing → AWS Lambda

- Event detection → AWS IoT Events

- Alerts and notifications → Amazon SNS

- System monitoring and logging → Amazon CloudWatch