# Sensor-to-Edge Data Architecture
This document describes the data architecture and communication interface between the STM32N6 microcontroller-based sensing system and the NVIDIA Orin NX edge AI processor.

# The system is designed to collect data from multiple sensing modalities, process and structure the data at the microcontroller level, transmit the processed sensor data through a Wi-Fi communication interface, and perform edge AI inference to compute quality attributes such as freshness, aroma, spoilage risk, and other product characteristics.

The system consists of three major processing layers:

| Layer | Component | Function |
| --- | --- | --- |
| Sensing Layer | Sensors (CH0–CH6) | Collect environmental, chemical, visual, and physical signals |
| Embedded Layer | STM32N6 MCU | Sensor acquisition, sampling, preprocessing, aggregation |
| Edge AI Layer | NVIDIA Orin NX | Sensor fusion, AI inference, quality evaluation |

# The system follows a distributed processing model:

### STM32N6 Responsibilities

# Sensor acquisition

# Sampling

# Basic preprocessing

# Data aggregation

# Feature preparation

# Wireless transmission

### Orin NX Responsibilities

# Sensor fusion

# AI model inference

# Quality attribute computation

# Data storage and analytics

# Sensor Data Collection

| Channel | Sensor | Data Collected / Output |
| --- | --- | --- |
| CH0 | 2 kg Straight-Bar Load Cell | Force / Weight measurement |
| CH0 | MCU ADC Input / Interface | Analog voltage signals from sensors |
| CH1 | Sensirion SGP41 VOC/NOx Module | VOC Index, NOx Index, raw gas signal |
| CH1 | Bosch BME688 Module | Gas resistance, temperature, humidity, pressure |
| CH1 | Figaro TGS2602 VOC / Odor Sensor | Gas resistance, odor intensity response |
| CH1 | Renesas ZMOD4410 | Total VOC concentration, IAQ index, gas resistance |
| CH2a | Sensirion SCD41 CO₂ Sensor | CO₂ concentration, temperature, humidity |
| CH2a | MQ-136 Sulfur Gas Sensor | Hydrogen sulfide (H₂S) proxy concentration, sensor resistance |
| CH2a | MQ-135 Gas Sensor | Ammonia (NH₃) proxy, NOx proxy, alcohol vapors, benzene/smoke proxy |
| CH2a | Ethylene Electrochemical Sensor | Ethylene concentration, electrochemical current |
| CH2b | Figaro TGS2600 Air Contaminants Sensor | Hydrogen proxy, air contaminant concentration, sensor resistance |
| CH2b | PID VOC Module | Total VOC concentration, ionization current |
| CH3 | Sensirion SHT45 RH/T Sensor | Temperature, relative humidity |
| CH3 | MPXV7002DP Differential Pressure Sensor | Differential pressure, airflow estimation |
| CH4 | PMS7003 PM Sensor | PM1, PM2.5, PM10 concentrations, particle count |
| CH4 | Sensirion SPS30 PM Sensor | PM1, PM2.5, PM4, PM10 mass concentration, particle number concentration |
| CH5 | OV9281 Global-Shutter Camera | Image frames, shape, texture, color/visual features |
| CH5 | Fixed LED Illumination Set | Controlled illumination level for image capture |
| CH6 | AS7341 Spectral Sensor | Multi-band visible spectral intensities |
| CH6 | AS7265x Triad Spectroscopy Module | 18-band spectral reflectance measurements |
| CH7 | 5 kg Load Cell + HX711 Kit | Weight, load cell voltage signal |
| CH7 | MPU6050 / Vibration Sensor | Acceleration (X,Y,Z), angular velocity |
| CH8 | INA219 Current / Power Monitor | Voltage, current, power consumption |
| CH8 | DS18B20 Temperature Probe | Temperature |
| CH8 | Reed Switch Module | Door/chamber open–close status |
| CH8 | SFM3003 Flow Sensor | Airflow rate |
| CH8 | 12V 1/8” NC Solenoid Valve | Airflow control state (open/close) |
| CH8 | 12V Diaphragm Pump | Air sampling flow generation |
| CH8 | Pump / Valve Driver Electronics | Actuator control signals |
| CH9 | INMP441 I²S MEMS Microphone | Acoustic signal / chamber sound |
| CH9 | VL53L1X ToF Sensor | Distance / fill-height measurement |

# Sensors Grouped by Sensor Type

| Sensor Modality | Sensors Included | Data Collected | Aggregated Data Sent to Edge (Parameters) |
| --- | --- | --- | --- |
| VOC sensing | SGP41, ZMOD4410, BME688, TGS2602, | VOC index, NOx index, total VOC concentration, ionization current | VOC_vector |
| MOS/MOX gas sensing | MQ135, MQ136, TGS2600,SCD41 (CO₂) | Gas resistance, odor response, ammonia proxy, sulfur proxy, air contaminant levels | MOS_vector |
| Electrochemical sensing | Ethylene EC sensor | Ethylene gas concentration, electrochemical current | EC_vector |
| Selective gas sensing | SCD41 (CO₂) | CO₂ concentration, temperature, humidity | EC_vector / Gas_vector |
| Environmental sensing | SHT45, DS18B20 | Temperature, humidity | ENV_vector |
| Airflow / pressure sensing | MPXV7002DP, SFM3003 | Differential pressure, airflow rate | ENV_vector |
| Particulate sensing | PMS7003, SPS30 | PM1, PM2.5, PM4, PM10 concentrations, particle count | PM_vector |
| Spectral sensing | AS7341, AS7265x | Multi-band spectral intensities, reflectance spectrum | SPECTRAL_vector |
| Vision sensing | OV9281 | Image frames, color, texture, object shape | VISION_data |
| Physical sensing | Load cells | Weight / force measurements | PHYSICAL_data |
| Motion sensing | MPU6050 | Acceleration, angular velocity | PHYSICAL_data |
| Platform health sensing | INA219 | Voltage, current, power consumption | SYSTEM_health_vector |
| Distance sensing | VL53L1X | Distance / fill height measurement | PHYSICAL_data |
| Acoustic sensing | INMP441 | Acoustic signal / sound profile | AUDIO_data |

# MCU to Edge data transmission

# ​​

Communication protocols that can be used:

| Protocol | Use case |
| --- | --- |
| MQTT | best for sensor streaming |
| TCP socket | simple direct connection |
| HTTP REST | request-response |
| UDP | low latency |

Mostly we will be going with MQTT





Data Reception on Orin NX
The Orin NX runs a server application

Example:-

The MCU will not only send sensor data, but also receive control commands from the Edge through the Wi-Fi module.

Instead of hardcoding the sampling period in STM32 firmware, the Edge AI system sends configuration commands such as:

- sampling period

- sensor enable/disable

- sampling window size

- start/stop acquisition

We will be using MQTT protocol for both side

| Direction | Topic | Purpose |
| --- | --- | --- |
| STM32 → Edge | mxsense/device01/data | sensor data |
| Edge → STM32 | mxsense/device01/control | commands |

Json that we will be sending to edge from stm32n6 

{

"header": {

"device_id": "MX-ORIN-NX-001",

"timestamp": "2026-03-17T02:05:00Z",

"firmware_version": "2.1.0-stable",

"location_tag": "Factory-Floor-A"

},

"mcu_monitoring": {

"carrier_board_mcu": "STM32-H7",

"uptime_seconds": 86400,

"vcc_main_v": 12.05,

"i2c_bus_status": "OK",

"spi_bus_status": "OK",

"watchdog_resets": 0

},

"device_health": {

"soc_temp_c": 45.5,

"gpu_load_percent": 62.0,

"cpu_load_percent": [15, 22, 10, 12, 18, 14],

"ram_usage_mb": 4102,

"emmc_storage_free_gb": 14.2,

"power_mode": "15W_6CORE",

"fan_speed_rpm": 2200

},

"sensor_payload": {

"env": {

"sht45_temperature": 0.0,

"sht45_humidity": 0.0,

"bme688_temperature": 0.0,

"bme688_humidity": 0.0,

"bme688_pressure": 0.0

},

"voc": {

"sgp41_voc_index": 0,

"sgp41_nox_index": 0,

"zmod4410_voc_concentration": 0.0,

"bme688_gas_resistance": 0.0,

"tgs2602_odor_level": 0.0

},

"gas": {

"mq135_air_quality": 0.0,

"mq136_sulfur_level": 0.0,

"tgs2600_contamination": 0.0,

"ethylene_sensor_value": 0.0

},

"pm": {

"pms7003_pm1": 0.0,

"pms7003_pm2_5": 0.0,

"pms7003_pm10": 0.0,

"sps30_pm1": 0.0,

"sps30_pm2_5": 0.0,

"sps30_pm10": 0.0

},

"spectral": {

"as7341_channels": [],

"as7265x_channels": []

},

"force": {

"loadcell_2kg_force": 0.0,

"loadcell_5kg_force": 0.0

},

"flow": {

"mpxv7002dp_pressure_diff": 0.0,

"sfm3003_flow_rate": 0.0

},

"system": {

"ds18b20_temperature": 0.0,

"ina219_current": 0.0,

"ina219_power": 0.0,

"reed_switch_state": false

},

"acoustic": {

"inmp441_noise_level": 0.0

},

"distance": {

"vl53l1x_fill_height": 0.0

}

},

"image_metadata": {

"camera_id": "CSI-01",

"frame_id": 98442,

"resolution": "1920x1080",

"format": "JPEG",

"storage_path": "/opt/mxsense/data/img/98442.jpg",

"inference_applied": true,

"detected_objects": []

}

}

*approx

Json that we will be sending to stm32n6 from edge in order to control different sensors

{

"header": {

"device_id": "stm32_01",

"timestamp": "2026-03-17T10:35:00Z",

"command_id": "cmd_001"

},

"mode_control": {

"operation_mode": "sampling"

},

"sampling_config": {

"global_interval_ms": 1000,

"env_interval_ms": 1000,

"voc_interval_ms": 2000,

"gas_interval_ms": 2000,

"pm_interval_ms": 5000,

"spectral_interval_ms": 10000,

"vision_interval_ms": 20000

},

"sensor_control": {

"sgp41_enabled": true,

"bme688_enabled": true,

"zmod4410_enabled": true,

"tgs2602_enabled": true,

"mq135_enabled": true,

"mq136_enabled": true,

"tgs2600_enabled": true,

"sps30_enabled": true,

"pms7003_enabled": false,

"as7341_enabled": true,

"as7265x_enabled": false,

"camera_enabled": true

},

"actuator_control": {

"pump_state": true,

"valve_state": false,

"purge_duration_ms": 5000

},

"calibration": {

"zero_calibration": false,

"span_calibration": false

}

}

Basically we will be sending in this way
STM32 → (sensor JSON) → Orin

Orin → (control JSON) → STM32

DATABASE SCHEMA DESIGN

STM32 → Orin (raw) → Orin processing → Backend API → Database

Data storage pipeline

Step 1: Data Acquisition (STM32 → Orin)

- STM32 collects high-frequency raw sensor data (100 ms – 1 s).

- Data is transmitted to Orin in structured JSON format.

- Data is stored temporarily in edge memory (buffer).

## Step 2: Edge Processing (Orin)

- Perform feature extraction:

  - Example: Average, min, max, standard deviation

  - Trends (increasing/decreasing)

- Perform data aggregation:

  - Multiple raw samples → single summarized record

- Perform anomaly detection:

  - Identify abnormal patterns or threshold violations

## Step 3: Data Segregation

Orin generates three types of outputs:

### 3.1 Processed Data (Primary)

- Aggregated and meaningful features

- Used for dashboards and analytics

### 3.2 Sampled Raw Data (Secondary)

- Downsampled sensor readings (e.g., every 1–5 seconds)

- Used for plotting graphs

### 3.3 Alerts / Events

- Generated on anomalies or threshold breaches

- Includes severity and type

## Step 4: Data Transmission (Orin → Backend)

- Data is sent to backend via:

  - REST API or MQTT(to be decided)

- Separate endpoints for:

  - Processed data

  - Sampled data

  - Alerts

## Step 5: Backend Processing

- Validate incoming data

- Attach metadata (timestamps, device ID if required)

- Route data to appropriate database tables

## Step 6: Data Storage (Database)

Data is stored in three main tables:

- Processed Data Table

  - Stores aggregated features and scores

- Sampled Sensor Data Table

  - Stores downsampled readings for visualization

- Alerts Table

  - Stores anomaly and event information

## Step 7: Data Retention Strategy

- Raw data (Edge): Short-term (buffer only)

- Sampled data: Medium-term (days/weeks)

- Processed data: Long-term (analytics and reporting)

# Database Creation on AWS (Amazon RDS – PostgreSQL)

## Overview
The cloud database is deployed using Amazon RDS (Relational Database Service) with PostgreSQL.

This provides a managed, scalable, and secure relational database for storing processed and sampled sensor data.

Step 1: Access AWS RDS

- Log in to AWS Console

- Navigate to RDS Dashboard

- Click on “Create Database”

## Step 2: Select Database Engine

- Engine type: PostgreSQL

- Version: Latest stable version (default)

Step 3: Choose Deployment Type

- Template: Free Tier / Dev-Test

- DB Instance Class: db.t3.micro (cost-efficient)

Step 4: Configure Database Settings

- DB Instance Identifier: mxsense-db

- Master Username: postgres

- Password: (securely configured)

- Initial Database Name: mxsense

Step 5: Configure Storage

- Storage Type: General Purpose (gp2/gp3)

- Allocated Storage: 20 GB (default)

Step 6: Configure Connectivity

- Virtual Private Cloud (VPC): Default

- Public Access: Enabled (for development)

- Security Group: Default (updated in next step)

Step 7: Create Database Instance

- Click “Create Database”

- Wait for the instance status to become Available

Step 8: Configure Network Access

- Navigate to Connectivity & Security

- Open the associated VPC Security Group

- Add inbound rule:

  - Type: PostgreSQL

  - Port: 5432

  - Source: 0.0.0.0/0 (for development only)

Step 9: Connect to Database

Use a database client (pgAdmin, DBeaver, etc.) with:

- Host: <RDS Endpoint>

- Port: 5432

- Username: postgres

- Password: <configured password>

- Database: mxsense

Step 10: Initialize Database Schema

- Execute the SQL schema scripts:

  - devices

  - processed_data

  - sampled_sensor_data

  - alerts

  - image_metadata

Step 11: Verify Setup

- Insert test data into tables

- Run queries to confirm successful setup